- TCG for shear wave calibration

- open the setup toolbox and click TCG bar

- move with the probe to the first FBH(the close on to the surface). With the motion function (vb/w/ and get the maximum respond from the FBH)

- click on the TCG bar      to create the first TCG point.

  - drag the green dot and pot it on the first pulse and set it on at 40% FSH .

  - Set the amplitude to 80% FSH

- move with the probe to the second FBH (the middle one to the surface). With the motion function (vb/w/ and get the maximum respond from the FBH)

  - click on the TCG bar      to create the second TCG point

  - drag the green dot and put it on the second pulse. and set it on at 40% FSH Set the amplitude to 80% FSH

- move with the probe to the third FBH (the last one to the surface). With the motion function (vb/w/ and get the maximum respond from the FBH)

  - click on the TCG bar      to create the third TCG point

  - drag the green dot and put it on the third pulse. and set it on at 40% FSH

  - Set the amplitude to 80% FSH

- Save